# Algorithm

Normal Condition

Helmet

↓

Read sensors

↓

Filter sensor noise

↓

Validate sensors

↓

Send packet

│

▼

Receive Sensor Packet

↓

Validate Data

↓

Normalize Using Worker Baseline

↓

Calculate Physiological Deviations

↓

Update Exposure Duration

↓

Generate Feature Vector

↓

Run ML Model

↓

Prediction Confidence Check

↓

Prediction Smoothing (last 3–5 predictions)

↓

Risk State Changed?

↓

YES

↓

Generate Alert

↓

Update Dashboard

↓

Send LED Command to Helmet

│

▼

Update LED

PART 1: Helmet Processing

The helmet is not responsible for deciding whether a worker is Safe, Warning, or Critical. Its responsibility is to collect accurate sensor data and communicate with the backend.

Stage 1: Read Sensors

- The helmet continuously acquires raw measurements from all integrated sensors.

- Input: Physical environmental and physiological conditions.

- Processing: Each sensor is read according to its sampling interval. These are raw sensor values. No decisions are made yet

Example:

Heart Rate = 92 BPM

Body Temperature = 36.8°C

Ambient Temperature = 34°C

Noise = 83 dB

Gas = 120 ppm

UV = 5

Sensors

| Sensor | Measures |
| --- | --- |
| MQ2 | Gas concentration |
| DHT11 | Ambient temperature & humidity |
| LM35 | Body temperature |
| MAX30102 | Heart rate |
| GUVA UV Sensor | UV exposure |
| INMP441 | Noise level |
| GPS | Worker location |

Stage 2: Filter Sensor Noise

- Real sensors naturally produce noisy and unstable measurements.

- Instead of transmitting unstable values, the helmet smooths the readings.

Example:

Raw heart rate

91

93

140

92

94

The value 140 BPM may simply be a sensor artifact caused by slight movement.

After filtering

91

92

93

93

94

Typical Filters

For your project, suitable filters include

- Moving Average Filter

- Exponential Moving Average (EMA)

- Median Filter

Your current firmware already performs

- Heart rate averaging

- LM35 smoothing

These can remain unchanged.

Stage 3: Validate Sensors

- Ensure that sensor readings are valid before transmitting them.

- This prevents faulty sensors from affecting the ML model.

Validation Examples

Heart Rate

Finger detected?

YES

Otherwise

Heart Rate = Invalid

Body Temperature

Is temperature between 32°C and 45°C?

If not

Invalid

GPS

Has GPS acquired a valid location?

Gas Sensor

ADC value within expected operating range?

- Each sensor is marked as Valid or Invalid. Only valid values are used.

Stage 4: Send Packet

Package all validated sensor readings into a JSON object and send them to the backend every 60 seconds.

Example Packet

{

"helmetId":"BS-H-001",

"workerId":"W-015",

"timestamp":"2026-07-05T09:45:00",

"heartRate":92,

"bodyTemp":36.8,

"ambientTemp":34,

"noise":83,

"gas":120,

"uv":5,

"gps":{

"lat":6.92,

"lon":79.85

}

}

PART 2: Backend Processing

The backend is responsible for all intelligent decision making.

Stage 5: Receive Sensor Packet

Receive incoming sensor packets from multiple helmets.

Before processing, the backend first identifies:

- Worker

- Helmet

- Time

Example

Helmet: BS-001

↓

Worker: Kamal

Stage 6: Validate Data

Although the helmet validates the data, the backend performs an additional validation step. This is to protect the system against:

- corrupted packets

- missing values

- communication errors

- invalid timestamps

Example

Heart Rate = NULL

↓

Reject packet

Another example

Body Temp = 120°C

↓

Discard value

This improves reliability.

Stage 7: Normalize Using Worker Baseline

Different workers naturally have different physiological characteristics.

For example

Worker A

Resting HR: 60 BPM

Worker B

Resting HR: 92 BPM

A heart rate of 100 BPM means

- Worker A: Large increase

- Worker B: Almost normal

Therefore the backend retrieves each worker's baseline.

Example

Worker: Kamal

↓

Baseline HR: 72 BPM

↓

Baseline Body Temp: 36.5°C

Stage 8: Calculate Physiological Deviations

Instead of using raw heart rate values, the backend calculates the percentage deviation from the worker's baseline.

Example

Heart Rate

Current: 100 BPM

Baseline: 80 BPM

Deviation: ((100-80)/80)*100 = 25%

Similarly, Body Temperature

Current: 38°C

Baseline: 36.5°C

Deviation: 4.1%

These deviations become inputs to the ML model.

Stage 9: Update Exposure Duration

Track how long abnormal conditions persist.

Example

Noise: 90 dB - 1 second -> Probably harmless.

But

90 dB - 20 minutes -> High risk.

The backend stores

Noise Exposure Duration = 1200 seconds

Similarly

Heart Rate Deviation - 25% -> Duration 8 minutes

Duration becomes another ML feature.

Stage 10: Generate Feature Vector

The backend combines all processed information into a single feature vector.

Example

Heart Rate :95

HR Deviation: 21%

Body Temp: 37.4

Body Temp Deviation: 3%

Noise: 84

Gas: 150

Ambient Temp: 34

UV: 6

Noise Exposure Duration: 320 sec

HR Exposure Duration: 180 sec

This feature vector becomes the ML model input.

Stage 11: Run ML Model

The feature vector is fed into the trained machine learning model.

Example

Random Forest or XGBoost

The model predicts

SAFE or WARNING or CRITICAL

Unlike threshold rules, the model considers all features simultaneously.

Stage 12: Prediction Confidence Check

Most ML models produce probabilities.

Example

Safe 5%

Warning 92%

Critical 3%

The prediction is very confident.

Another

Safe 45%

Warning 40%

Critical 15%

The model is uncertain.

The backend may decide to wait for another packet before updating the worker's status.

This helps reduce false positives.

Stage 13: Prediction Smoothing

Single predictions may occasionally fluctuate.

Example

SAFE

↓

WARNING

↓

SAFE

↓

WARNING

Instead of changing the worker's status every minute, the backend stores the last few predictions.

Example

SAFE

SAFE

WARNING

WARNING

WARNING

Majority: WARNING

Therefore the final state becomes WARNING. This prevents frequent changes caused by temporary fluctuations.

Stage 14: Risk State Changed?

The backend compares the current risk state with the previous one.

Example

Previous : SAFE

Current: SAFE

Nothing changes.

Another example

Previous : SAFE

Current: WARNING

A state transition has occurred.

The backend now generates an alert.

Stage 15: Generate Alert

The backend records the event.

Example

Time: 09:35

Worker: Kamal

Helmet: BS-001

Status; WARNING

Notifications may also be sent

- Dashboard

- SMS

- Email

- Push notification

depending on your implementation.

Stage 16: Update Dashboard

The dashboard updates immediately.

Example

Helmet BS-001

🟡 WARNING

The supervisor can now

- View live sensor readings.

- View historical trends.

- See the worker's location.

- View AI-generated recommendations.

- Acknowledge or resolve the alert.

Stage 17: Send LED Command to Helmet

Finally, the backend instructs the helmet to display the appropriate LED color.

Example

{

"risk":"WARNING"

}

Helmet

↓

Yellow LED

The helmet does not calculate the risk itself. It simply follows the backend's instruction.

Emergency Condition

Helmet

↓

Emergency=true

↓

Every 3 s

↓

GET /helmet/emergency/reset

↓

Nothing?

↓

Continue

When backend replies

{
   "reset":true
}

Helmet:

Emergency=false
↓
LED controlled by backend again
↓
Stop polling

PART 3: Emergency Workflow

Emergency handling is completely separate from the ML workflow because it represents a direct request for assistance.

Stage 1: Emergency Button Pressed

The worker presses the emergency button.

The helmet immediately:

- Changes the LED to red.

- Sets its internal state to EMERGENCY.

- Sends an emergency packet to the backend immediately, without waiting for the normal 60-second interval.

Stage 2: Backend Receives Emergency

The backend immediately:

- Records the emergency event.

- Displays the worker as EMERGENCY on the dashboard.

- Notifies the safety officer.

- Displays the worker's GPS location.

- Activates the Reset Emergency control in the web application.

No ML processing is required because the worker explicitly requested help.

Stage 3: Emergency Reset

Once the situation has been verified, the supervisor presses Reset Emergency in the web application.

The backend sends a reset command to the helmet.

While the helmet is in the EMERGENCY state, it periodically listens for this reset command. After receiving a valid reset:

- The helmet clears the emergency state.

- Stops listening for further reset commands.

- Returns to normal sensor monitoring.

- Resumes following LED commands from the backend based on ML predictions.